# Roode Calibration – Adaptive Heuristics Addendum

This addendum replaces fixed heuristic numbers with adaptive, robust defaults. It can be applied on top of your current v2 docs without changing architecture.

## Parameter Defaults & Safe Ranges

| Key | Default | Safe Range | Purpose |
| --- | --- | --- | --- |
| mcps_floor_strategy | max(5, p10(MCPS)) | min clamp 5 | Adaptive floor for “dark” SPADs |
| top_fraction_for_center | auto (15–30%) | 0.15–0.30 | Fraction of SPADs used for weighted center |
| stable_mask_rule | MCPS ≥ median + 0.5·MAD | MAD factor 0.3–0.8 | Robust stability criterion |
| variance_cv_mask | 0.25 | 0.15–0.35 | Mask SPADs with coefficient of variation above this |
| trials_per_config | 3 (bump to 5 if noisy) | 3–5 | Number of trials per grid/mode |
| trial_bump_cv | 0.20 | 0.15–0.25 | Raise trials if average CV above this |
| scan_time_cap_seconds | 90 | 60–180 | Skip 16×16 if total scan time exceeds cap |
| min_threshold_strategy (distance) | p95(idle_dist) | 92–96% of idle median | Lower detection threshold |
| max_threshold_strategy (signal) | SNR target 7 (simple fallback 80% max MCPS) | SNR 6–8 (70–85%) | Upper threshold + hysteresis guard |
| hysteresis_min_gap | 10% of (max−idle) | 8–12% | Ensure max ≥ min + gap |
| mode_tie_break | prefer lower timing budget | — | Tie‑break when variances equal |
| new_session_half_life_days | 14 | 7–28 | Weight recent sessions more |
| cluster_size_ratio_max | 4.0 | 3–6 | Reject k=2 split if too imbalanced |

## Algorithms (Concise)

### 1) Adaptive MCPS Floor

floor = max(5, percentile(MCPS_all, 10))
keep SPAD if MCPS ≥ floor

### 2) Stable SPAD Mask (Robust)

med = median(MCPS_roi)
MAD = median(|MCPS - med|)
keep if MCPS ≥ med + 0.5*MAD AND CV ≤ 0.25

### 3) Top‑Fraction for Center Weighting

N = number of SPADs in ROI
k = clamp( (64.0 / N) * 0.05, 0.15, 0.30 )   # fraction
use_top = top k fraction of SPADs by MCPS for centroid
center = weighted_centroid(use_top)

### 4) Trials & Noise Adaptation

if mean(CV_trials) > 0.20: trials_per_config = 5 else 3

### 5) Grid Scan Time Guard

if elapsed_scan_time > scan_time_cap_seconds:
    skip remaining 16x16 passes

### 6) Thresholds

Min (distance based):

min_thr = percentile(distance_idle, 95)
min_thr = clamp(min_thr, 0.92*idle_median, 0.96*idle_median)

Max (signal based preferred):

choose max_thr such that SNR ≥ target_snr (default 7)
if SNR unavailable: max_thr = 0.80 * MCPS_max
max_thr ≥ min_thr + 0.10*(MCPS_max - MCPS_idle)

### 7) Ranging Mode Selection

mode = argmin(variance_over_trials)
if tie: pick mode with smaller timing budget (short/med)

### 8) Entry/Exit Split

project SPAD coords onto door axis → 1D
k=2 clustering on 1D values
discard split if (max(cluster_size)/min(cluster_size)) > 4.0
if discarded: retry with different ROI size or adjacent axis

### 9) Multi‑Session Weighting

weight(session_age_days) = exp( - age / new_session_half_life_days )

## Firmware Hooks (ESPHome)

- Add optional constants (with defaults above) as compile‑time or runtime parameters.

- Expose variance_cv_mask, trial_bump_cv, scan_time_cap_seconds as read‑only sensors for telemetry (optional).

- Continue publishing raw MCPS/distance so HA can apply rules.

## HA Analysis Hooks

- Implement strategies exactly as above; store chosen values in roi_result.json.

- Make defaults user‑tunable via UI (advanced section) but hidden under a “Show expert parameters” toggle.

## Minimal Config Block (YAML‑like)

calibration:
  mcps_floor_strategy: percentile10_clamped_5
  top_fraction_for_center: auto_15_30
  stable_mask:
    mad_factor: 0.5
    variance_cv_mask: 0.25
  trials:
    default: 3
    bump_if_cv_over: 0.20
  scan_time_cap_seconds: 90
  thresholds:
    min:
      source: idle_distance_p95
      clamp: [0.92x_idle_median, 0.96x_idle_median]
    max:
      source: snr_target
      snr_target: 7
      fallback_percent_of_max_mcps: 0.80
      hysteresis_gap: 0.10
  mode_tie_break: prefer_lower_timing_budget
  history:
    new_session_half_life_days: 14
  zones:
    cluster_size_ratio_max: 4.0

Drop‑in Usage: Place this addendum after the existing v2 docs. Implement HA‑side logic first (no firmware change required if you already publish MCPS/distance). Optionally mirror key parameters on the device for observability.