Roode Calibration: Step 1 & 2 Implementation Guide (Enhanced)

🔍 Scope

This updated guide incorporates enhancements compatible with the Roode repository (codex/update-version-to-1.7.0 branch). It extends the original Step 1 (Passive Scanning) and Step 2 (ROI Auto Selection) with automated ROI and zone setting logic, leveraging the VL53L1X’s ROI programming capabilities. The goal is to reduce manual configuration by automatically determining ROI size, center, and entry/exit zones.

This version assumes use of ESPHome with VL53L1X drivers and Home Assistant for orchestration.

✅ Step 1: Passive Grid Scanning

Purpose: Collect MCPS and distance data from multiple ROI grid sizes and ranging modes to establish a baseline environmental signal profile.

ESPHome Responsibilities: - Accept trigger from HA (input_select.roode_calibration_control or custom service) - Perform scans in the following pattern: - ROI sizes: 4x4, 8x8, 16x16 - Each ROI scanned 3 times (trials) - For each ROI, scan all ranging modes: short, medium, long - Package each scan as structured JSON (MCPS + distance matrix) - Emit scan_complete signal once all scans are done

Output Format (per scan):

{
  "type": "roi_scan",
  "grid": "8x8",
  "trial": 2,
  "ranging": "short",
  "timestamp": "2025-08-01T17:01:00Z",
  "data": [[mcps_00, ..., mcps_07], ..., [mcps_70, ..., mcps_77]]
}

Home Assistant Responsibilities: - Create new calibration session folder: calibration_2025-08-01_17-00/ - Collect and store all scans in session.json - Wait for scan_complete before triggering analysis

✅ Step 2: ROI Auto Selection

Purpose: Automatically determine optimal ROI center, size, and entry/exit zones based on passive scan data.

Inputs: - session.json containing all ROI sizes, ranging modes, and trials

Plugin Actions: 1. Merge Scans: - Average MCPS per SPAD index over 3 trials per grid/mode

- Noise Filtering:

  - Discard SPADs with MCPS < 10 (too dark)

  - Mask SPADs with high variance

- ROI Selection Logic:

  - Compute ROI center = weighted average of high-MCPS SPADs

  - ROI size = bounding box of clustered stable zones

  - Select best ROI (grid + ranging mode) with most stable readings

- Zone Assignment:

  - Split ROI into two clusters (entry and exit)

  - Compute centroids for each cluster

  - Assign one as zones.entry.center and the other as zones.exit.center

- Threshold and Mode Tuning:

  - Set lower threshold = 90–95% of idle distance

  - Set upper threshold = 60–80% of peak MCPS

  - Select ranging mode with lowest variance and strongest signal

Output: - Store result in roi_result.json in same session folder - Apply results back to Roode automatically via OTA YAML update or custom API call: - roi_width, roi_height - zones.entry.center, zones.exit.center - ranging_mode - Threshold percentages

📊 Auto-Calculated Setting Table

| Setting | Description | Formula / Logic |
| --- | --- | --- |
| roi_center | Center SPAD of highest activity cluster | Weighted average of top 20% MCPS SPADs |
| roi_width, roi_height | Optimal bounding box size | Range of stable SPADs > 50% median MCPS |
| spad_mask | Zones to ignore | SPADs with MCPS < 10 or high variance |
| zones.entry.center | Entry zone ROI center | Stable high-MCPS cluster (door side A) |
| zones.exit.center | Exit zone ROI center | Opposite stable cluster (door side B) |
| detection_thresholds.min | Lower threshold | 90–95% of idle distance |
| detection_thresholds.max | Upper threshold | 60–80% of highest MCPS |
| sampling | Averaging level | Increase if MCPS variance high |
| filter_window | Movement smoothing window | Larger = smoother, slower |
| ranging_mode | Chosen ranging mode | Most stable, strong signal across trials |

📂 Storage & File Handling

Each calibration session = single folder, e.g. calibration_2025-08-01_17-00/ - session.json: raw scan data - roi_result.json: computed ROI + thresholds

Multiple sessions are stored in /calibration/; the latest sessions are weighted higher in analysis.

🌐 Communication Flow

From HA to Roode: - Trigger via input_select.roode_calibration_control with options: - start_passive_scan - analyze_roi

From Roode to HA: - JSON messages containing scan results - Status signal scan_complete

🚀 Enhancement Summary

This enhanced guide extends the original calibration plan by: - Adding automatic ROI size and zone assignment - Using passive scan data to auto-set thresholds - Eliminating manual ROI/zone config in YAML - Applying results back to the device automatically via OTA/API

This ensures Roode optimally configures itself with minimal user intervention, while remaining compatible with the existing ESPHome + Home Assistant integration.